NEUROTECH NOC

Autonomous Network Operations Agent

System Features, Logic & Tech Stack

Complete reference guide for Manan Shah, CTO — Neurotech

What this system does Neurotech NOC is an AI-powered autonomous agent that continuously watches your entire ISP network, detects problems before they cascade into outages, reasons about root causes using Claude AI, and takes corrective action — automatically for safe fixes, with operator approval for riskier ones.

# 1. The Core Loop: Observe → Reason → Decide → Act → Learn

Everything in this system flows through a single 5-stage loop, running every 2 seconds continuously. This is what makes it an agent rather than a simple monitoring dashboard.

| Stage | What Happens | Who/What Does It |
| --- | --- | --- |
| OBSERVE | Collects live telemetry from all 10 network nodes — latency, packet loss, utilization, health score | telemetry.py — TelemetrySimulator |
| REASON | Detects statistical anomalies, checks thresholds, identifies correlated failures across nodes | anomaly.py — AnomalyDetector |
| DECIDE | Calls Claude AI with full network context; Claude forms a hypothesis and proposes 3 tiered actions | agent.py — NetworkAgent + Claude API |
| ACT | Executes Tier 1 actions automatically; queues Tier 2 for operator approval; escalates Tier 3 to engineers | main.py — Action Executor |
| LEARN | Logs every action + outcome; tracks success rates; adjusts cooldown timers to reduce false alerts | main.py — action_history / agent stats |

| KEY | The loop runs in the background on the server — the frontend just receives real-time updates via WebSocket. Claude AI is only called when an anomaly is confirmed, not on every tick. |
| --- | --- |

# 2. Feature Breakdown

## 2.1  Live Network Topology Map

An interactive SVG map showing all 10 nodes and 11 links in real time. Node colors update every 2 seconds based on live telemetry.

| What you see | What it means |
| --- | --- |
| Green node | Healthy — all metrics within normal range |
| Amber node | Warning — utilization > 85% OR packet loss > 0.5% OR health < 88 |
| Red node | Critical — utilization > 105% OR packet loss > 1.5% OR health < 75 |
| Dashed red link | The link between two nodes is heavily congested (avg util > 90%) |
| Pulsing ring | This node was just flagged by an anomaly detection event |
| Utilization % label | Real-time traffic load on that node as a percentage of capacity |

Node types and their roles:

- Core (CORE-ATL-01, CORE-NYC-01) — backbone routers, largest circles, highest capacity links

- Edge (EDGE-ATL-01/02, EDGE-NYC-01/02) — aggregation layer between core and PoPs

- Peer (PEER-IX-01/02) — Internet Exchange peering points, where external traffic enters/exits

- PoP (POP-ATL-01, POP-NYC-01) — Points of Presence, closest to end customers

## 2.2  Anomaly Detection Engine

Three independent detection methods run on every telemetry tick. Any node that trips one or more becomes a candidate for AI analysis.

| Method | How It Works | What It Catches |
| --- | --- | --- |
| Threshold Breach | Compares current value to static limits per node type. Core nodes have tighter limits than PoP nodes. | Sustained overload, packet loss spikes, health degradation |
| Z-Score (Statistical) | Measures how many standard deviations the current value is from the rolling 20-sample mean. Triggers at Z > 3.0. | Sudden spikes that are abnormal for that specific node's history |
| Rate of Change | Calculates % change between the last two readings. Triggers if any metric jumps > 50% in one tick. | Rapid onset events: fiber cuts, BGP flaps, DDoS absorption |
| Composite Pattern | Checks whether multiple related nodes are anomalous simultaneously. Named patterns: congestion cascade, BGP instability, fiber degradation. | Multi-node correlated failures invisible to per-node checks |

| NOTE | A 6-tick cooldown prevents the same node from generating repeated alerts. Composite patterns have a 10-tick cooldown. This is the 'learn' component — it self-adjusts based on how noisy the network is. |
| --- | --- |

## 2.3  Claude AI Reasoning Engine

When an anomaly is confirmed, the agent packages full network context and sends it to Claude (claude-sonnet-4). Claude responds with structured JSON — not a free-text reply.

What Claude receives in every request:

- Current metrics on all affected nodes (latency, packet loss, utilization, health)

- Last 5 telemetry samples for each affected node (trend data — is it getting worse?)

- The anomaly type and all indicator signals that triggered it

- Full network-wide snapshot of all 10 nodes

- A detailed system prompt giving Claude expert ISP network engineering context

What Claude returns:

- Hypothesis — technical explanation of what the signals indicate

- Root Cause — most probable single cause with supporting evidence

- Risk Score — 0 to 100, factoring in blast radius and customer impact

- Confidence — how certain Claude is (0 to 100%)

- Affected Customers Estimate — estimated subscriber impact

- Time to Impact — estimated time until customer-visible outage if unaddressed

- 3 Actions — one per tier, each with detail, rollback instructions, and ETA

| FALLBACK | If the Anthropic API is unavailable (no key set or network error), the agent falls back to a rule-based response. The frontend continues working — it just shows generic reasoning instead of Claude's analysis. |
| --- | --- |

## 2.4  Tiered Action System (Guardrails)

Every proposed action is classified into one of three tiers based on blast radius, reversibility, and customer impact. This is the safety model.

| Tier | Execution | Example Actions |
| --- | --- | --- |
| Tier 1 — AUTO | Executes automatically within ~0.5 seconds, no human required. Only actions that are fully reversible and cannot cause an outage. | ECMP weight rebalancing, BGP soft route suppression, traffic pre-positioning, metric pre-adjustment, increasing monitoring frequency |
| Tier 2 — APPROVE | Queued in the frontend Action Queue. A blinking badge shows the count. Operator clicks APPROVE or REJECT. Only then does the backend execute it. | QoS policy changes, BFD timer adjustments, rate limiting policies, controlled failover to backup path |
| Tier 3 — ESCALATE | Never auto-executed. Shown as AWAITING NOC in the UI. In production this would trigger a PagerDuty alert or Slack message to the on-call engineer. | Emergency capacity activation, physical fiber inspection, full IX peer failover, core routing changes |

Every action — regardless of tier — records:

- Execution timestamp

- Outcome (success / failed)

- A rollback snapshot — so it can be reversed with one click

- Execution log — CLI-style line-by-line record of what happened

## 2.5  Real-Time WebSocket Feed

The frontend does not poll the backend. Instead, a persistent WebSocket connection (ws://localhost:8000/ws) streams all events as they happen. This means zero latency between detection and display.

| Event Type | When It Fires |
| --- | --- |
| telemetry | Every 2 seconds — updated metrics for all 10 nodes |
| agent_state | When the agent transitions: observing → reasoning → acting |
| incident | When Claude completes analysis of an anomaly |
| action_executed | When a Tier 1 action auto-executes or operator approves Tier 2 |
| action_rejected | When operator rejects a proposed action |
| action_rolled_back | When rollback is triggered on a previously executed action |
| init | On first connection — full current state sync |

If the WebSocket disconnects (server restart, network blip), the frontend automatically attempts reconnection every 3 seconds. The header shows LIVE / CONNECTING / OFFLINE status.

## 2.6  Scenario Injection Panel

The Scenarios tab lets you manually trigger any of 4 realistic ISP failure scenarios into the backend simulator. This is primarily for testing and demonstration — Claude AI will respond to each one as if it were a real incident.

| Scenario | What It Simulates |
| --- | --- |
| Atlanta Congestion Cascade | Evening traffic surge overloads POP-ATL-01. Utilization spikes +38%, latency +22ms on EDGE-ATL-01/02. Tests ECMP rebalancing and QoS actions. |
| BGP Route Flap NYC | BFD timer mismatch causes repeated BGP session resets on PEER-IX-02. Packet loss +2.2%. Tests protocol-layer detection and BGP soft suppression. |
| Fiber Degradation ATL-Core | Progressive optical power drop on CORE-ATL-01. Health degrades -28 points over time. Tests predictive failure detection before full outage. |
| DDoS Absorption Event | Large volumetric attack absorbed across all 4 IX/core nodes simultaneously. Tests composite pattern detection and multi-node coordination. |

After injecting a scenario, the telemetry simulator applies the changes gradually (not instantly) with slight randomness, making the signals look realistic rather than abrupt.

# 3. Under the Hood — How the Logic Works

## 3.1  Telemetry Simulation (telemetry.py)

The simulator uses mean-reverting random walks — each metric drifts toward its baseline value over time with added Gaussian noise. This produces realistic-looking time-series data.

Formula per metric per tick:

new_value = current + (baseline − current) × 0.05 + Gaussian(0, noise_scale)

The 0.05 reversion factor means if a metric is pushed high by a scenario, it will naturally drift back toward normal over ~60 ticks — mimicking how real network conditions self-correct after a transient event.

A diurnal factor (sin wave over 24 hours) modulates baseline utilization and latency to simulate realistic peak-hour traffic patterns — highest at 20:00, lowest at 04:00.

## 3.2  Anomaly Scoring and Cooldowns (anomaly.py)

Each node builds a rolling window of its last 20 telemetry samples. The Z-score is calculated fresh on every tick:

Z = |current_value − window_mean| / window_std_dev

A Z-score above 3.0 means the reading is 3 standard deviations away from that node's recent normal — statistically very unlikely to be random noise. Above 4.5 is classified as critical.

Cooldown timers prevent alert storms: after a node fires an anomaly, it is suppressed for 6 ticks (~12 seconds). This is stored in a per-node dictionary and decrements on each tick. Composite patterns use a 10-tick cooldown.

## 3.3  Claude AI Prompt Engineering (agent.py)

The system prompt gives Claude a specific identity and strict output rules. Claude is told to respond ONLY with valid JSON — no preamble, no markdown, no explanation outside the schema. If Claude adds markdown fences, the parser strips them.

The user message includes three types of context deliberately:

- Current state — what is happening right now on affected nodes

- Trend data — the last 5 readings, so Claude can say 'degrading rapidly' vs 'stable at elevated level'

- Global state — all 10 nodes, so Claude can identify which healthy nodes could absorb rerouted traffic

Claude is also given explicit safety instructions in the system prompt: never recommend a Tier 1 action with blast radius > 2, always include a rollback strategy, and factor customer count into the risk score calculation.

## 3.4  Action Execution and Rollback

In the current version, action execution is simulated (with realistic timing delays and a 5% failure rate to test rollback logic). In production you would replace the execute_action() function body with:

- NETCONF/RESTCONF calls to your routers for configuration changes

- gRPC calls for telemetry and real-time parameter updates

- REST API calls to your NMS/orchestration layer for higher-level operations

- PagerDuty or Slack webhooks for Tier 3 escalations

Every executed action is appended to action_history (never deleted) and given a unique ID. The rollback endpoint uses this ID to find the action and reverse it.

# 4. Tech Stack Breakdown

## 4.1  Frontend

| Technology | What It Does Here |
| --- | --- |
| React 18 | Component framework. Single-file App.jsx with hooks for all state management. |
| Vite | Development server and build tool. Provides instant hot-reload when you edit App.jsx. |
| WebSocket API (native) | Built-in browser API. Maintains persistent connection to the backend for real-time event streaming. |
| Fetch API (native) | Built-in browser API. Used for REST calls: approve action, reject action, rollback, trigger scenario. |
| SVG (inline in React) | Network topology map drawn entirely in SVG — no external charting library needed. |
| CSS-in-JS (inline styles) | All styling done via React's style prop and a single injected <style> tag. No CSS files, no Tailwind dependency. |
| JetBrains Mono / Syne | Google Fonts loaded at runtime. Monospace for telemetry data, Syne for headings and numbers. |

## 4.2  Backend

| Technology | What It Does Here |
| --- | --- |
| Python 3.10+ | Primary backend language. All 5 backend files are pure Python. |
| FastAPI | Web framework. Handles all REST endpoints and the WebSocket connection. Auto-generates Swagger docs at /docs. |
| Uvicorn | ASGI server that runs FastAPI. Supports async operations needed for the background telemetry loop and WebSocket broadcasting. |
| asyncio | Python's async library. The telemetry loop, WebSocket broadcast, and action execution are all async — they run concurrently without blocking. |
| Pydantic v2 | Data validation for REST request bodies. ActionRequest model validates incoming approve/reject requests. |
| python-dotenv | Loads ANTHROPIC_API_KEY from the .env file into the environment on startup. |

## 4.3  AI / Intelligence Layer

| Technology | What It Does Here |
| --- | --- |
| Anthropic Python SDK | Official SDK for calling the Claude API. Handles authentication, request formatting, and error handling. |
| Claude (claude-sonnet-4) | The AI model doing the actual reasoning. Chosen for its strong technical knowledge, structured output reliability, and fast response time. |
| Structured JSON output | Claude is prompted to return only valid JSON. The parser strips markdown fences and validates the schema before using the response. |
| Z-score anomaly detection | Custom implementation in anomaly.py. Rolling 20-sample window per node. No ML framework needed — pure Python math. |
| Composite pattern matching | Rule-based correlation engine. Checks if predefined node groups are all anomalous simultaneously to identify multi-node failure patterns. |
| Mean-reverting random walk | Telemetry simulation model. Gaussian noise + reversion toward baseline. Produces realistic time-series without requiring real network hardware. |

## 4.4  Communication Layer

| Technology | What It Does Here |
| --- | --- |
| WebSocket (FastAPI native) | Persistent bidirectional connection. Backend pushes telemetry, incidents, and action results without the frontend needing to poll. |
| REST API (FastAPI) | 7 endpoints for actions the frontend triggers explicitly: approve, reject, rollback, scenario injection, stats retrieval. |
| CORS middleware | Allows the React dev server (localhost:5173) to call the FastAPI server (localhost:8000) without browser security blocks. |
| JSON | All data exchanged as JSON — telemetry snapshots, incident objects, action records, WebSocket events. |

## 4.5  What You Would Add for Production

| Component | Production Replacement |
| --- | --- |
| telemetry.py simulator | SNMP polling (pysnmp), gNMI/gRPC streaming from routers, sFlow/NetFlow collectors |
| In-memory action_history list | PostgreSQL with SQLAlchemy ORM — persists across server restarts |
| In-memory telemetry_history deque | InfluxDB or TimescaleDB — purpose-built time-series databases with compression and query support |
| Simulated execute_action() | NETCONF via ncclient library, RESTCONF calls, or your NMS vendor API |
| No authentication | JWT tokens on REST endpoints, WSS (WebSocket Secure) with token auth |
| Basic anomaly detection | LSTM or Transformer trained on your historical telemetry for more accurate predictions |
| Manual Tier 3 display | PagerDuty API, Slack webhooks, or OpsGenie for real on-call alerting |

# 5. API Quick Reference

## 5.1  REST Endpoints

| Method + Path | What It Does | Used By |
| --- | --- | --- |
| GET / | Health check — returns {status: ok} | Monitoring |
| GET /telemetry/current | Live snapshot of all 10 nodes | Frontend on load |
| GET /telemetry/history?limit=100 | Rolling telemetry history array | Graphing tools |
| GET /incidents | All detected incidents with full detail | Frontend sidebar |
| GET /agent/stats | Success rate, action count, incident count | Footer stats |
| POST /actions/approve | Execute an operator-approved Tier 2 action | Approve button |
| POST /actions/reject | Reject and discard a proposed action | Reject button |
| POST /actions/rollback/{id} | Reverse a previously executed action | Rollback button |
| POST /simulate/scenario/{id} | Inject a failure scenario into the simulator | Scenarios tab |
| WS /ws | Real-time bidirectional event stream | Frontend WebSocket |

## 5.2  Scenario IDs

| ID | Scenario Name |
| --- | --- |
| congestion-atl | Atlanta Congestion Cascade |
| bgp-flap | BGP Route Flap NYC |
| fiber-cut | Fiber Degradation ATL-Core |
| ddos | DDoS Absorption Event |

NEUROTECH NOC v2.0 Observe → Reason → Decide → Act → Learn